AUTOMATIC CURTAIN OPENER

PROJECT REPORT ON ELECTRONICS PART-ANTON OUSEPH(B240143CS)

1. Project Overview:

This project involves designing and building an automated curtain opener system using NodeMCU (ESP8266) as the core controller. The system enables motorized control of window blinds through a web interface, and voice commands via a custom mobile app(MIT APP INVENTOR). It also includes safety features such as limit switches to prevent mechanical damage.

2. Objectives:

- Enable wireless (Wi-Fi) control of blinds using a mobile app.

- Implement limit switches to stop the motor at end positions.

- Provide voice control using App Inventor’s SpeechRecognizer.

- Minimize component count using the built-in features of NodeMCU.

3. Components Used:

| Component | Quantity | Purpose |
| --- | --- | --- |
| NodeMCU (ESP8266) | 1 | Microcontroller & Wi-Fi web server |
| 12V DC Motor | 1 | Drives the blinds mechanism |
| L298N Motor Driver Module | 1 | Controls motor direction and speed |
| Limit Switches | 2 | Detect end positions (fully open/close) |
| 12V Power Adapter(but we used external power source because 12V battery was not available) | 1 | Powers the motor and L298N |
| Jumper Wires | Several | Circuit connections |
| MIT App Inventor App |  | Sends commands via Wi-Fi and voice |

4. Working Principle:

- The NodeMCU acts as a Wi-Fi access point and hosts a web server.

- When a user sends a command (via web or app), NodeMCU reads it and activates the motor through the L298N motor driver.

- Limit switches are used to detect end positions and automatically stop the motor to prevent over-driving.

- App Inventor also integrates voice control via the SpeechRecognizer module, which converts voice to command and sends it as an HTTP request.

5. Circuit Connections:

NodeMCU to L298N:

| NodeMCU Pin | L298N Pin | Function |
| --- | --- | --- |
| D1 | IN1 | Motor Direction 1 |
| D2 | IN2 | Motor Direction 2 |
| D3 | ENA | Speed control (PWM) |
| VIN | 12V IN | Power for L298N (through external adapter) |
| GND | GND | Common ground |

Limit Switches:

| NodeMCU Pin | Component | Function |
| --- | --- | --- |
| D5 | Upper Limit Switch | Stops motor at full open |
| D6 | Lower Limit Switch | Stops motor at full close |

6. Power Supply Design:

- Motor Power: A 12V battery /external power source  is used to drive the DC motor through the L298N module.

- Logic Power: NodeMCU is powered via USB .

7. Control Logic:

The control commands include:

- /open → Activates motor to rotate in the opening direction.

- /close → Activates motor to rotate in the closing direction.

- /stop → Stops the motor.

Limit Switch Logic:

- If a limit switch is pressed (LOW), the motor is stopped to prevent further movement in that direction.

8. Advantages:

- Wireless control via smartphone browser or app.

- Independent of external Wi-Fi (NodeMCU creates its own network).

- Safe operation using limit switches.

- Voice command integration.

- Can be upgraded with sensors or smart home platforms (like Blynk or Home Assistant).

9. Conclusion:

The electronics of the smart vertical blinds system are designed for flexibility, reliability, and ease of use. Using a NodeMCU greatly simplifies the wiring and logic by integrating Wi-Fi control, allowing for smart automation and seamless integration with mobile apps and voice commands.

10.Referneces:

- Language Reference | Arduino Documentation

- https://youtu.be/2xDbvp7mCCY

- https://randomnerdtutorials.com/esp8266-pinout-reference-gpios/

- I have created a basic plan for this project in tinkercad.( https://www.tinkercad.com/things/5FlShbO3NZz-basic-circuit?sharecode=fQz1Jyf5xZ95EQHkSRDwf7agZnjM9dvUkiawX3f0-dY )